# DA_Analysis_Summary

2026-08-04

## Introduction

Accotink Creek near Annandale, VA (Gage 01654000) has a drainage area (DA) of 23.9 square miles and has recorded flow data from 1947-2026.Accotink Creek is a tributary of the Potomac River that feeds Accotink Lake, a reservoir in Fairfax County, VA.A case study on 7/06/2026 revealed that this gage has only two identified baseflow events. This is a very flashy gage; the low flows make identifying a baseflow start dates difficult, and predictions are very sensitive to start dates. With only two baseflow events, flow predictions from the regression must be made with considerable caution.

These observations at Accotink Creek presented the question: do other “small gages” (those with a drainage area less than 50 square miles) have limited baseflow events? Ultimately, does the current model work at smaller gages, or are there aspects to the baseflow workflow that need to be modified to address the flashy behavior often seen at these small gages?

Virginia gages with similar DA and date range to Accotink were collected and recorded in Table 1:

Table 1. List of small gages in Virginia with available flow dates and drainage area similar to Accotink Creek

| Gage Location | Gage ID | DA (mi2) | Start Date | End Date | # BF Events | Land Use |
| --- | --- | --- | --- | --- | --- | --- |
| S F Quantico Creek near Independent Hill, VA | 01658500 | 7.62 | 1951 | 2026 | 5 | National Forest, limited urbanization |
| Tinker Creek near Daleville, VA | 02055100 | 11.70 | 1956 | 2026 | 78 | Mostly urbanized |
| Fourmile Run at Alexandria, VA | 01652500 | 12.60 | 1951 | 2026 | No gage file found | Dense urbanization |
| Fine Creek at Fine Creek Mills, VA | 02036500 | 22.4 | 1944 | 2026 | 5 | Some farmland and forest, some urbanization |
| Battle Run near Laurel Mills, VA | 01662800 | 25.8 | 1958 | 2026 | 25 | Mostly farmland, some forest, limited urbanization |
| Beaver Creek at Bristol, VA | 03478400 | 26.9 | 1957 | 2026 | 83 | Some farmland, some urbanization |
| Piscataway Creek near Tappahannock, VA | 01669000 | 27.9 | 1951 | 2026 | 19 | Mostly farmland and forest, limited urbanization |
| Catawba Creek near Catawba, VA | 02018500 | 34.3 | 1943 | 2026 | 43 | Mostly farmland and forest, limited urbanization |
| Kerrs Creek near Lexington, VA | 02022500 | 35.1 | 1927 | 2026 | 47 | Mostly farmland and forest, limited urbanization |
| Chestnut Creek at Galax, VA | 03165000 | 39.4 | 1944 | 2026 | 44 | Mostly farmland and forest, limited urbanization |

## Methods

## Results

## Discussion

## Conclusion